2023 Consumer Confidence Report for Public Water System FORT GRIFFIN SUD

| This is your water quality report for January 1 to December 31, 2022 |  | For more information regarding this report contact: |
| --- | --- | --- |
| FORT GRIFFIN SUD provides Purchased Surface Water from City of Albany (Hubbard Lake), located in Albany, Texas |  | Tyler George |
|  |  | (325)762-2575 |

Este reporte incluye información importante sobre el agua para tomar.  Para asistencia en español, favor de llamar al telefono (325)762-2575.

Definitions and Abbreviations

| Definitions and Abbreviations |  | The following tables contain scientific terms and measures, some of which may require explanation. |
| --- | --- | --- |
| Action Level: |  | The concentration of a contaminant which, if exceeded, triggers treatment or other requirements which a water system must follow. |
| Avg: |  | Regulatory compliance with some MCLs are based on running annual average of monthly samples. |
| Level 1 Assessment: |  | A Level 1 assessment is a study of the water system to identify potential problems and determine (if possible) why total coliform bacteria have been found in our water system. |
| Level 2 Assessment: |  | A Level 2 assessment is a very detailed study of the water system to identify potential problems and determine (if possible) why an E. coli MCL violation has occurred and/or why total coliform bacteria have been found in our water system on multiple occasions. |
| Maximum Contaminant Level or MCL: |  | The highest level of a contaminant that is allowed in drinking water. MCLs are set as close to the MCLGs as feasible using the best available treatment technology. |
| Maximum Contaminant Level Goal or MCLG: |  | The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs allow for a margin of safety. |
| Maximum residual disinfectant level or MRDL: |  | The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants. |
| Maximum residual disinfectant level goal or MRDLG: |  | The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants. |
| MFL |  | million fibers per liter (a measure of asbestos) |
| mrem: |  | millirems per year (a measure of radiation absorbed by the body) |
| na: |  | not applicable. |
| NTU |  | nephelometric turbidity units (a measure of turbidity) |
| pCi/L |  | picocuries per liter (a measure of radioactivity) |

Definitions and Abbreviations

| ppb: |  | micrograms per liter or parts per billion |
| --- | --- | --- |
| ppm: |  | milligrams per liter or parts per million |
| ppq |  | parts per quadrillion, or picograms per liter (pg/L) |
| ppt |  | parts per trillion, or nanograms per liter (ng/L) |
| Treatment Technique or TT: |  | A required process intended to reduce the level of a contaminant in drinking water. |

Information about your Drinking Water

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity.

Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that water poses a health risk.  More information about contaminants and potential health effects can be obtained by calling the EPAs Safe Drinking Water Hotline at (800) 426-4791.

Contaminants that may be present in source water include:

-   Microbial contaminants, such as viruses and bacteria, which may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife.

-   Inorganic contaminants, such as salts and metals, which can be naturally occurring or result from urban storm water runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming.

-   Pesticides and herbicides, which may come from a variety of sources such as agriculture, urban storm water runoff, and residential uses.

-   Organic chemical contaminants, including synthetic and volatile organic chemicals, which are by-products of industrial processes and petroleum production, and can also come from gas stations, urban storm water runoff, and septic systems.

-   Radioactive contaminants, which can be naturally occurring or be the result of oil and gas production and mining activities.

In order to ensure that tap water is safe to drink, EPA prescribes regulations which limit the amounts of certain contaminants in water provided by public water systems. FDA regulations establish limits for contaminants in bottled water which must provide the same protection for public health.

Contaminants may be found in drinking water that may cause taste, color, or odor problems.  These types of problems are not necessarily causes for health concerns.  For more information on taste, odor, or color of drinking water, please contact the system's business office.

You may be more vulnerable than the general population to certain microbial contaminants, such as Cryptosporidium, in drinking water.  Infants, some elderly, or immunocompromised persons such as those undergoing chemotherapy for cancer; persons who have undergone organ transplants; those who are undergoing treatment with steroids; and people with HIV/AIDS or other immune system disorders, can be particularly at risk from infections. You should seek advice about drinking water from your physician or health care providers.  Additional guidelines on appropriate means to lessen the risk of infection by Cryptosporidium are available from the Safe Drinking Water Hotline (800-426-4791).

If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. We are responsible for providing high quality drinking water, but we cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. If you are concerned about lead in your water, you may wish to have your water tested. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline or at http://www.epa.gov/safewater/lead.

Information about Source Water

FORT GRIFFIN SUD purchases water from CITY OF ALBANY. CITY OF ALBANY provides purchase surface water from City of Albany (Hubbard Lake) located in Albany, Texas.

TCEQ completed a Source Water Susceptibility for all drinking water systems that own their sources. This report describes the susceptibility and types of constituents that may come into contact with the drinking water source based on human activities and natural conditions. The system(s) from which we purchase our water received the assessment report. For more information on source water assessments and protection efforts at our system contact Fort Griffins SUD at (325)762-2575].

| Lead and Copper | Date Sampled | MCLG | Action Level (AL) | 90th Percentile | # Sites Over AL | Units | Violation | Likely Source of Contamination |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Copper | 2022 | 1.3 | 1.3 | 0.168 | 0 | ppm | N | Erosion of natural deposits; Leaching from wood preservatives; Corrosion of household plumbing systems. |
| Lead | 2022 | 0 | 15 | 0 | 1 | ppb | N | Corrosion of household plumbing systems; Erosion of natural deposits. |

2023 Water Quality Test Results

2023

| Disinfection By-Products | Collection Date | Highest Level Detected | Range of Individual Samples | MCLG | MCL | Units | Violation | Likely Source of Contamination |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chlorite | 2023 | 0.805 | 0.423 - 0.805 | 0.8 | 1 | ppm | N | By-product of drinking water disinfection. |
| Haloacetic Acids (HAA5) | 2023 | 16 | 11.8 - 16.7 | No goal for the total | 60 | ppb | N | By-product of drinking water disinfection. |

*The value in the Highest Level or Average Detected column is the highest average of all HAA5 sample results collected at a location over a year

| Total Trihalomethanes (TTHM) | 2023 | 43 | 20.8 | 44.1 | 80 | ppb | Y | By-product of drinking water disinfection. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

*The value in the Highest Level or Average Detected column is the highest average of all TTHM sample results collected at a location over a year

| Inorganic Contaminants | Collection Date | Highest Level Detected | Range of Individual Samples | MCLG | MCL | Units | Violation | Likely Source of Contamination |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Barium | 2023 | 0.18 | 0.18 - 0.18 | 2 | 2 | ppm | N | Discharge of drilling wastes; Discharge from metal refineries; Erosion of natural deposits. |
| Cyanide | 2023 | 67.4 | 67.4 - 67.4 | 200 | 200 | ppb | N | Discharge from plastic and fertilizer factories; Discharge from steel/metal factories. |
| Fluoride | 2023 | 0.3 | 0.276 - 0.276 | 4 | 4.0 | ppm | N | Erosion of natural deposits; Water additive which promotes strong teeth; Discharge from fertilizer and aluminum factories. |
| Nitrate [measured as Nitrogen] | 2023 | 0.233 | 0.174 - 0.233 | 10 | 10 | ppm | N | Runoff from fertilizer use; Leaching from septic tanks, sewage; Erosion of natural deposits. |

2023

| Radioactive Contaminants | Collection Date | Highest Level Detected | Range of Individual Samples | MCLG | MCL | Units | Violation | Likely Source of Contamination |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Beta/photon emitters | 06/09/2021 | 8.8 | 8.8 - 8.8 | 0 | 50 | pCi/L* | N | Decay of natural and man-made deposits. |

*EPA considers 50 pCi/L to be the level of concern for beta particles.

Disinfectant Residual

A blank disinfectant residual table has been added to the CCR template, you will need to add data to the fields. Your data can be taken off the Disinfectant Level Quarterly Operating Reports (DLQOR).

| Disinfectant Residual | Year | Average Level | Range of Levels Detected | MRDL | MRDLG | Unit of Measure | Violation (Y/N) | Source in Drinking Water |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chloramine Chlorine Dioxide | 2022 2022 | 2.25 0.06 | 0.0/3.7 0-0.74 | 4 .8 | 4 .8 | Ppm ppm | No No | Water additive used to control microbes. Water additive used to control microbes. |

Violations

| Chlorine |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Some people who use water containing chlorine well in excess of the MRDL could experience irritating effects to their eyes and nose. Some people who drink water containing chlorine well in excess of the MRDL could experience stomach discomfort. |  |  |  |  |  |
| Violation Type | Violation Begin |  | Violation End |  | Violation Explanation |
| Disinfectant Level Quarterly Operating Report (DLQOR). | 01/01/2022 |  | 03/31/2022 |  | We failed to test our drinking water for the contaminant and period indicated. Because of this failure, we cannot be sure of the quality of our drinking water during the period indicated. |

| Total Trihalomethanes (TTHM) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Some people who drink water containing trihalomethanes in excess of the MCL over many years may experience problems with their liver, kidneys, or central nervous systems, and may have an increased risk of getting cancer. |  |  |  |  |  |
| Violation Type | Violation Begin |  | Violation End |  | Violation Explanation |
| MCL, LRAA | 10/01/2022 |  | 12/31/2022 |  | Water samples showed that the amount of this contaminant in our drinking water was above its standard (called a maximum contaminant level and abbreviated MCL) for the period indicated. |

Regarding the Violations noted above:

“ Disinfectant Level Quarterly Operating Report (DLQOR) 01/01/2022-03/31/2022

We failed to test our drinking water for the contaminant and period indicated. Because of this failure, we cannot be sure of the quality of our drinking water during the period indicated.”

The drinking water WAS tested for the contamination period indicated. All testing was done on time and results were within the acceptable standards set forth by the state for healthy drinking water. FGSUD is required to submit test results to the state quarterly and these results were not received by the deadline, which resulted in this violation.  This was simply a reporting issue and had nothing to do with the quality of the water.  However, because results were not received by the deadline, TCEQ could not attest to the quality of water for the period indicated.

City of Albany Water Quality Test Results

| 2023 | Chloroform | 1.6 | 1.12 | 2.07 | ppb | Byproduct of drinking water disinfection |
| --- | --- | --- | --- | --- | --- | --- |
| 2023 | Bromoform | 15.57 | 3.86 | 26.27 | ppb | Byproduct of drinking water disinfection |
| 2023 | Bromodichloromethane | 3.97 | 2.04 | 4.72 | ppb | Byproduct of drinking water disinfection |
| 2023 | Dibromochloromethane | 9.33 | 4.32 | 13.2 | ppb | Byproduct of drinking water disinfection |

Cryptosporidium Monitoring Information

Not yet tested for. Cryptosporidium, a microbial parasite that might be commonly found in surface water. Cryptosporidium may come from animal and human feces in the watershed.

| Year | Inorganic Contaminants | Highest Level Detected | Range of Levels Detected | MCLG | MCL | Units | Violation | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2022 | Arsenic | 1 | 1 | 0 | 10 | ppb | No | Erosion of natural deposits; runoff from orchards, glass production and electronic production waste |
| 2023 | Barium | .18 | .18 | 2 | 2 | ppm | No | Discharge from drilling waste; discharge from metal refineries; erosion of natural deposits. |
| 2023 | Fluoride | .3 | .276 | 4 | 4 | ppm | No | Erosion of natural deposits; additive which promotes teeth; discharge from fertilizer and aluminum factories. |
| 2023 | Nitrate [measured as Nitrogen] | .233 | .178-233 | 10 | 10 | ppm | No | Runoff from fertilizer used; leaching from septic tanks, sewage and erosion of natural deposits. |
| 2022 | Selenium | 5 | 5 | 50 | 50 | ppb | No | Discharge from petroleum and metal refineries; erosion of natural deposits; discharge from mines. |
| 2022 | Chromium | 2.4 | 2.4 | 100 | 100 | ppb | No | Discharge from steel and pulp mills; Erosion of natural deposits. |
| 2023 | Cyanide | 67.4 | 67.4 | 200 | 200 | ppb | No | Discharge from plastic and fertilizer factories; Discharge from steel/metal factories. |

Nitrate Advisory: Nitrate in drinking water at levels above 10 ppm is a health risk for infants of less than six months of age. High nitrate levels in drinking water can cause Blue Baby Syndrome. Nitrate levels may rise quickly for short periods of time because of rainfall or agricultural activity. If you are caring for an infant you should ask advice from your health care provider.

| Year | Disinfection Byproducts | Highest Level Detected | Range of Levels | MCLG | MCL | Unit | Violations | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2023 | Haloacetic Acids | 16.7 | 11.8-16.7 | No goal for the total | 60 | ppb | No | By-product of drinking water Disinfection. |
| 2023 | Total Trihalomethanes | 44 | 20.8-44.1 | No goal for the total | 80 | ppb | No | By-product of drinking water Disinfection. |
| 2023 | Chlorite | .805 | .423-.805 | .8 | 1 | ppm | No | By-product of drinking water Disinfection |

| Year | Radioactive Contaminants | Highest Level Detected | Range of Levels Detected | MCLG | MCL | Units | Violation | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 06/09/2021 | Gross Beta Emitters | 8.8 | 8.8 | 0 | 50 | pCi/L* | No | Decay of natural and man- made Deposits. |

*EPA considers 50 pCi/L to be the level of concern for beta particles

Unregulated Contaminants

Bromoform, Chloroform, Bromodichloromethane, and Dibromochloromethane are disinfectant byproducts. There is no maximum contaminant level for these chemicals at the entry point to the distribution system.

| Year | Contaminant | Average Level | Minimum Level | Maximum Level | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| 2023 | Chloroform | 1.6 | 1.12 | 2.07 | ppb | Byproduct of drinking water disinfection |
| 2023 | Bromoform | 15.57 | 3.86 | 26.27 | ppb | Byproduct of drinking water disinfection |
| 2023 | Bromodichloromethane | 3.97 | 2.04 | 4.72 | ppb | Byproduct of drinking water disinfection |
| 2023 | Dibromochloromethane | 9.33 | 4.32 | 13.2 | ppb | Byproduct of drinking water disinfection |

Lead and Copper

| Lead and Copper | Collection Date | MCLG | Action Level | 90th Percentile | Number of sites over AL | Units | Violation | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lead | Sept. 2021 | 0 | 15 | 0 | 0 | ppb | No | Corrosion of household plumbing system; erosion of natural deposits; leaching from wood preservatives. |
| Copper | Sept. 2021 | 1.3 | 1.3 | 0 | 0 | ppm | No | Corrosion of household plumbing system; erosion of natural deposits; leaching from wood preservatives |

Additional Health Information for Lead

“If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. This water supply is responsible for providing high quality drinking water but cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. If you are concerned about lead in your water, you may wish to have it tested. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline or at http://www.epa.gov/safewater/lead.

| Year | Turbidity | Highest Single Measurement | Percent of samples meeting the limit | Limit | Unit of Measure | Violations | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2023 | Turbidity | .05 | 100 | .3 | NTU | No | Soil runoff |

Information Statement: Turbidity is a measure of the cloudiness of the water caused by suspended particles. We monitor it because it is a good indicator of water quality and the effectiveness of our filtration process.

Coliform Bacteria

| MCLG | Total Coliform and E.coli | Number of Samples collected | Fecal Coliform Maximum Contaminant Level | Number of Positive Samples | Violations | Likely Source of Contamination |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Total Coliform | 24 | NA | 0 | No | Naturally present in the environment |
| 0 | E.coli | 24 | NA | 0 | No | Naturally present in the environment |

Total Organic Carbon

Total Organic Carbon (TOC) no health effects. The disinfectant can combine with TOC to form disinfection byproducts. Disinfection is necessary to ensure that water does not have unacceptable levels of pathogens. Byproducts of disinfection include trihalomethanes (THMs) and haloacetic acids (HAA) which are reported elsewhere in this report.

| Year | Contaminant | Average | Minimum | Maximum | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- |
| 2023 | Source Water | 5.48 | 5.12 | 6.06 | ppm | Naturally occurring |

Disinfectant Level

| Year | Disinfectant Used | Average | Range of Detection (low/High) | MRDL | MCDL | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2023 | Chloramines | 3.0 | 0.0/3.4 | 4 | <4 | ppm | Disinfectant used to control microbes |
| 2023 | Chlorine Dioxide | 0.06 | 0/0.74 | .8 | .8 | ppm | Disinfectant used to control microbes |

Secondary and Other Non-Regulated Constituents (No associated adverse health effects)

| Year | Constituent | Average | Minimum | Maximum | Limit | Unit of Measure | Source of Contaminant |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2023 | Bicarbonate | 122 | 122 | 122 | NA | ppm | Corrosion of carbonate rocks such as limestone |
| 2023 | Calcium | 53.5 | 53.5 | 53.5 | NA | ppm | Abundant naturally occurring element |
| 2023 | Chloride | 139 | 139 | 139 | 300 | ppm | Abundant naturally occurring element; used in water purification and byproducts of oilfield activity |
| 2023 | Iron | 0.05 | 0.05 | 0.05 | .3 | ppm | Erosion of natural deposits; iron or steel water delivery equipment or facilities |
| 2023 | Magnesium | 16.9 | 16.9 | 16.9 | NA | ppm | Abundant naturally occurring element |
| 2023 | Nickel | .001 | 0.001 | 0.001 | NA | ppm | Erosion of natural deposits |
| 2023 | pH | 7.8 | 7.4 | 8 | 7 | Units | Measure of corrosivity of water |
| 2023 | Sodium | 67.9 | 67.9 | 67.9 | NA | ppm | Erosion of natural deposits; byproduct of oilfield activity |
| 2023 | Sulfate | 58.9 | 58.9 | 58.9 | 300 | ppm | Naturally occurring; common industrial byproduct; byproduct of oilfield activity |
| 2023 | Total Alkalinity as CaC03 | 122 | 120 | 114 | NA | ppm | Naturally occurring soluble mineral salts |
| 2023 | Total Dissolved Solids | 415 | 415 | 415 | 1000 | ppm | Total dissolved mineral constituents in water |
| 2023 | Total hardness as CaC03 | 203 | 203 | 203 | NA | ppm | Naturally occurring calcium |

Interim Enhanced SWTR The Interim Enhanced Rule improves control of microbial contaminants, particularly Cryptosporidium, in systems using surface water, or ground water under the direct influence of surface water. The rule builds upon the treatment technique requirements of the Surface Water Treatment Rule.